Project info

Project title:	3_1_Situac_Ilgterm_Asni transports  A1_2024g Celi_Esosha

Project No.:

Project engineer:

Customer:

Description:

3_1_Situac_Ilgterm_Asni transports  A1_2024g Celi_Esosha

Run description

Calculation type:	Facade Noise Map

Title:         	Fasades

Calculation group

Run file:      	RunFile.runx

Result number:	4

Local calculation (ThreadCount=32)

Calculation start:	13.05.2025 15:53:02

Calculation end:	13.05.2025 15:53:05

Calculation time:	00:00:136 [m:s:ms]

No. of points:	24

No. of calculated points:	24

Kernel version:	SoundPLANnoise 9.1 (06.05.2025) - 64 bit

Run parameters

Reflection order: 	0

Maximum reflection distance to receiver 	200 m

Maximum reflection distance to source 	50 m

Search radius 	5000 m

Weighting:             	dB(A)

Allowed tolerance:	0,500 dB

Create ground effect areas from road surfaces: 	Yes

Treat roads as terrain following: 	No

Standards:

Road: 	NMPB 96

Driving on right side

Emission according to: 	Guide du Bruit

Limitation of screening loss:

single/multiple  	20,0 dB /25,0 dB

Side diffraction: disabled

Environment:

Air pressure 	1013,3 mbar

rel. humidity 	70,0 %

Temperature 	10,0 °C

Favorable / homogeneous percentages according to table:

Riga

Dissection parameters:

Distance to diameter factor 	8

Minimal distance                 	1 m

Max. difference ground effect + diffraction 	1,0 dB

Max. number of iterations 	4

Attenuation

Foliage:       	User defined

Built-up area: 	User defined

Industrial site:	User defined

Assessment: 	Latvija_2014

Facade Noise Map:

Distance to facade 	2,00 m

One receiver in center of facade

Reflection of "own" facade is suppressed

Reflections of objects from "own" property is suppressed

Geometry data

Situation1.sit	13.05.2025 15:52:06

- contains:

AprekinuDati.geo	30.04.2025 14:34:02

Asni_Koord.geo	14.11.2022 16:08:40

Ekas_Kadastrs_lv.geo	28.04.2025 10:23:04

GalvenaisIzvesanas_Cels_Ekas_Uztvereji_Ar_Fasadem.geo	13.05.2025 15:30:58

Teritor_Izmat_Funkcijas.geo	30.04.2025 09:56:34

Transp_Marsruts_1.geo	12.05.2025 10:19:06

Transp_Marsruts_2.geo	30.04.2025 14:34:02

VecstalesCela_Alternat_EkasUztv_Ar_Fasadem.geo	13.05.2025 15:52:06

Vegetacija.geo	28.04.2025 11:14:26

Zemes_ABS.geo	16.11.2022 18:43:48

Zogi.geo     	14.11.2022 12:01:46

RDGM0005.dgm 	25.04.2025 15:10:54